# DESIGN AND IMPLEMENTATION OF A SMALL-TO-MEDIUM ENTERPRISE NETWORK

(CCNA Individual Project – Track B)

## NAME:ADEJUWON PRECIOUS

## DEPARTMENT:COMPUTER SCIENCE

## MATRIC NUMBER:2021/10696

## CHAPTER ONE

## INTRODUCTION

### 1.1 Background of the Study

In today’s digital age, computer networks form the backbone of communication and information sharing within organizations. From small businesses to large enterprises, networking enables efficient data transfer, resource sharing, and access to internet services. A well-designed network improves productivity, ensures reliability, and supports organizational growth.

Small-to-medium enterprises (SMEs) often operate across multiple offices or branches and require a structured network that supports internal communication while remaining cost-effective and easy to manage. Networking technologies such as routers, switches, and dynamic addressing mechanisms play a crucial role in meeting these requirements.

This project focuses on the design and implementation of a small-to-medium enterprise network consisting of three branches using Cisco Packet Tracer. The project is aligned with CCNA curriculum objectives and emphasizes practical understanding of networking concepts such as IP addressing, routing, Dynamic Host Configuration Protocol (DHCP), and troubleshooting.

### 1.2 Aim and Objectives of the Project

The main aim of this project is to design and implement a functional enterprise network that demonstrates core CCNA networking concepts in a simulated environment.

The specific objectives of the project are to:

- Design a logical and physical network topology for a three-branch enterprise

- Configure routers and switches for effective communication

- Implement IP addressing schemes for different branches

- Configure DHCP for automatic IP address assignment

- Test network connectivity using standard diagnostic tools

- Introduce intentional network faults and perform troubleshooting

- Document the design, implementation, and troubleshooting processes

### 1.3 Scope of the Project

This project is limited to a simulated environment using Cisco Packet Tracer. It focuses on wired local area networks (LANs) within three branches. Advanced enterprise features such as firewall implementation, wireless networking, and real hardware deployment are beyond the scope of this project. However, the concepts implemented are scalable and applicable to real-world enterprise networks.

### 1.4 Significance of the Study

This project provides hands-on experience in enterprise network design and troubleshooting. It helps in developing practical skills required for network administration roles and prepares students for real-world networking scenarios. The project also reinforces theoretical knowledge gained from CCNA coursework.

## CHAPTER TWO

## LITERATURE REVIEW

### 2.1 Computer Networks

A computer network is a collection of interconnected devices that communicate and share resources using standardized protocols. Networks can be classified based on size, architecture, and purpose. Common network types include Local Area Networks (LANs), Wide Area Networks (WANs), and Metropolitan Area Networks (MANs).

### 2.2 Enterprise Networks

Enterprise networks are designed to support organizational operations across departments or locations. They emphasize reliability, scalability, and manageability. Typical enterprise networks use structured addressing schemes, centralized services such as DHCP, and routing mechanisms to connect multiple subnets.

### 2.3 Network Devices

#### 2.3.1 Routers

Routers are networking devices that connect different networks together. They operate at the network layer of the OSI model and make forwarding decisions based on IP addresses. Routers are essential for inter-network communication.

#### 2.3.2 Switches

Switches operate at the data link layer and are used to connect devices within the same network. They forward frames based on MAC addresses and provide efficient LAN connectivity.

### 2.4 Internet Protocol (IP) Addressing

IP addressing uniquely identifies devices on a network. IPv4 addressing uses a 32-bit address format and is commonly represented in dotted decimal notation. Proper IP addressing is essential for device communication and routing.

### 2.5 Dynamic Host Configuration Protocol (DHCP)

DHCP is a network protocol used to automatically assign IP addresses and related configuration information to devices. DHCP reduces manual configuration errors and simplifies network management.

## CHAPTER THREE

## SYSTEM DESIGN

### 3.1 Network Topology Design

The network is designed as a three-branch enterprise model. Each branch contains:

- One router

- One switch

- Two personal computers (PCs)

Each branch operates as an independent LAN with its own IP network.

### 3.2 Logical Network Design

Each branch is assigned a unique IP address range to avoid conflicts. Routers serve as default gateways for devices within their respective branches. Switches connect end devices within each LAN.

### 3.3 IP Addressing Scheme

| Branch | Network Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Branch 1 | 192.168.1.0 | 255.255.255.0 | 192.168.1.1 |
| Branch 2 | 192.168.2.0 | 255.255.255.0 | 192.168.2.1 |
| Branch 3 | 192.168.3.0 | 255.255.255.0 | 192.168.3.1 |

### 3.4 Physical Network Design

Physically, copper straight-through cables were used to connect PCs to switches and switches to routers. Router interfaces were enabled manually to ensure link connectivity.

## CHAPTER FOUR

## SYSTEM IMPLEMENTATION

### 4.1 Router Configuration

Each router interface was configured with an IP address corresponding to its branch network. Interfaces were activated using the no shutdown command to ensure operational status.

### 4.2 Switch Configuration

Switches were used as access layer devices to connect end-user PCs. Switch ports connecting to routers were enabled to allow data transmission between devices.

### 4.3 DHCP Implementation

DHCP was configured on each branch router. The DHCP pools provided:

- IP addresses

- Subnet masks

- Default gateways

- DNS server addresses

This allowed PCs to automatically obtain network configurations.

### 4.4 End Device Configuration

All PCs were configured to obtain IP addresses automatically using DHCP. This simplified network management and ensured consistency across devices.

## CHAPTER FIVE

## TESTING AND TROUBLESHOOTING

### 5.1 Network Testing

Network connectivity was tested using the ping command. Successful ping replies confirmed proper configuration and communication between devices.

### 5.2 Intentional Faults Introduced

Three faults were deliberately introduced to test troubleshooting skills:

#### Fault One: DHCP Failure

- Problem: PCs failed to receive default gateway

- Cause: Missing default-router command

- Solution: Reconfigured DHCP pool

#### Fault Two: Interface Shutdown

- Problem: Network link down

- Cause: Router interface shutdown

- Solution: Enabled interface using no shutdown

#### Fault Three: Incorrect IP Address

- Problem: PC could not communicate

- Cause: Wrong IP address assignment

- Solution: Corrected IP configuration

### 5.3 Troubleshooting Tools Used

- Ping command

- Interface status checks

- IP configuration verification

TROUBLE SHOOTING FAULTS

## Fault 1: DHCP Failure

## Problem: PCs could not access the network

## Cause: Missing default gateway in DHCP pool

## Solution: Reconfigured default-router command

## Fault 2: Interface Shutdown

## Problem: Network link was down

## Cause: Router interface was shutdown

## Solution: Enabled interface using no shutdown

## Fault 3: Wrong IP Address

## Problem: PC could not communicate with network

## Cause: Incorrect IP configuration

## Solution: Changed PC back to DHCP

## CHAPTER SIX

## RESULTS AND DISCUSSION

The implemented network successfully achieved its design objectives. DHCP simplified IP management, routing enabled device communication, and troubleshooting demonstrated practical problem-solving skills. The results confirm that the network design is functional, scalable, and aligned with CCNA standards.

## CHAPTER SEVEN

## CONCLUSION AND RECOMMENDATIONS

### 7.1 Conclusion

This project successfully demonstrated the design, implementation, and troubleshooting of a small-to-medium enterprise network using Cisco Packet Tracer. Core CCNA networking concepts were applied in a practical manner, reinforcing both theoretical and hands-on knowledge.

### 7.2 Recommendations

Future improvements may include:

- Implementation of VLANs

- Introduction of wireless networking

- Enhanced security features such as ACLs and firewalls

## REFERENCES

Cisco Networking Academy. (2023). Introduction to Networks. Cisco Press.
Forouzan, B. A. (2017). Data Communications and Networking. McGraw-Hill.